USB数据线规格书

USB data cable specification

产品名称：苹果数据线（D1）

Product name：：Apple Data Cable (D1)

制作时间：2022.12.3

Production time: 2022.12.3

目录

Directory

- 1、目的purpose……………………………………………………………………………… 3

- 2、适用范围Scope of application………………………………………………… 3

3、规格要求specifications…………………………………………………… 3

4、技术要求The technical requirements…………………………………… 5

4.1 使用环境Using the environment………………………………………… 5

4.2 数据线的插头与尺寸要求Plug and size requirements for data cables… 5

4.3 外观要求Cosmetic requirements………………………………………… 5

5、焊接要求The welding requirements…………………………………………… 5

6、功能测试A functional test…………………………………………………… 6

6.1 插拔测试Plug test………………………………………………………… 6

6.2拉力测试Tensile tests…………………………………………………… 6

6.3弯曲摇摆测试Bend and sway test……………………………………… 6

7、包装、储存和运输Packaging, storage and transportation……………… 7

7.1  包装packaging…………………………………………………………… 7

7.2  运输transport……………………………………………………………… 7

- 目的purpose

本承认书旨在提供客户作承认时的参考及依据。

（The purpose of this acknowledgement is to provide reference and basis for the client to make the acknowledgement.）

- 适用范围Scope of application

本规定指定的数据线外观结构、电气特性、机械方法、检验规格与质量要求，适用与为研发设计开发、认证测试以及品质来检验做参考依据。

（The appearance structure, electrical characteristics, mechanical methods, inspection specifications and quality requirements of the data lines specified in this regulation shall be used as reference basis for R&D, design and development, certification testing and quality inspection.）

- 规格要求specifications

3.1、使用的连接器：

Connectors used：

| 数据线 USB cable |  |  |
| --- | --- | --- |
| 管脚pin | A母 | 苹果插头 |
| 红red | PIN 1 | VCHG |
| 白white | PIN 2 | D- |
| 绿green | PIN 3 | D+ |
| 黑black | PIN 4 | GND |

3.2、线材示意图Wire Diagram

｜————————— 1000mm ±20mm—————————｜

4、技术要求The technical requirements

4.1使用环境（Operating environment）：

工作温度（working temperature）：—0℃~40℃

储运温度（Storage and transportation temperature）：—0℃~40℃

相对湿度（relative humidity）：≤93%[（40±2）℃

大气压力（atmospheric pressure）：70—106Kpa

4.2 数据线的插头与尺寸要求Plug and size requirements for data cables

手机端插头：手机插头采用苹果套壳插头，插头壳丝印EGA.两端都带塑胶保护套。确保其可靠性。

Mobile phone end plug: The mobile phone plug uses Apple case plug, plug shell screen printing EGA. Both ends are covered with plastic protective cover. Ensure its reliability.

尺寸要求：线材OD=2.9灰白色TPE线。数据线总线长要求1000mm。充电芯线采用40枝0.08的纯铜线, 下载芯线采用10枝0.08的纯铜线。插头端无线标。透明硬壳包装。

Size requirement: OD=2.9 gray TPE cable. The datacable bus length must be 1000mm. 40 0.08 copper wires are used for charging cores, and 10 0.08 copper wires are used for downloading cores. Plug end wireless beacon. Transparent hard case packing.

4.3 外观要求Cosmetic requirements

数据线不能有划伤，起泡、烫伤、露铜丝、缩水、异色等、输入输出插口正常，不可翘pin.

Data line can not have scratch, blister, scald, exposed copper wire, shrinkage, different color, etc., input and output jack is normal, can not warp pin.

5、焊接要求The welding requirements

针焊接要求：焊接不可短路。

Needle welding requirements: welding can not short circuit.

- 6、功能测试A functional test

6.1插拔测试Plug test

测试条件: 插拔频率每分钟15-30次，要求手机放在开机状态下测试。每1000次全面检查一次功能。

Test conditions: the frequency of insertion and removal is 15-30 times per minute, and the mobile phone is required to be placed in the switched on state for testing. Function is fully checked every 1000 times.

测试要求：1000次测试后功能无异常，手机插头端拔力至少0.1kgF,pin针不能有变形、脱针等不良、插头与母座不能有断裂和变形、塑胶插头不能严重磨损、起毛,母座功能不能损坏。

Test requirements: After 1000 tests, the function is normal, the pull force of the plug end of the mobile phone is at least 0.1kgF, the pin pin can not be deformed or removed, the plug and the mother seat can not be broken or deformed, the plastic plug can not be seriously worn and furred, and the mother seat function can not be damaged.

6.2拉力测试Tensile tests

测试条件：电线拉力要求至少要承受0.5 kgF拉力,时间一分钟。

Test conditions: The wire tension is required to withstand at least 0.5 kgF tension for one minute.

6.3弯曲摇摆测试Bend and sway test

测试条件：数据线两头0.25KgF的砝码，以-60度与+60度的角度每分钟来回摇摆30次一直做完3000次。

Test conditions: a 0.25KgF weight at both ends of the data line was swung back and forth 30 times per minute at an Angle of -60 degrees and +60 degrees until 3000 times were completed.

测试要求：测试后要求线材不能断或裂，不能有明显磨损、通话中用手摇晃两处接头不能有任何接触不良和功能异常。

Test requirements: After the test, it is required that the wire should not be broken or cracked, there should be no obvious wear, and the two joints should not have any poor contact or abnormal function when shaken by hand during the conversation.

- 7、包装、储存和运输Packaging, storage and transportation

7.1包装packaging

包装箱应贴有制造厂名、产品型号、产品名称、生产日期。

The packing case shall be affixed with the manufacturer's name, product model, product name and production date.

产品包装应防潮、防震。

Product packing should be moisture-proof and shockproof.

7.2运输transport

产品运输中应防雨淋、水浸。不应有剧烈震动。

Products should be protected from rain and water during transportation. There should be no violent vibration.